Basic Four Series 2000
  Desktop Computer System
       Error Logging Facility
          Technical User’s Guide

Basic Four® Series 2000

Desktop Computer System

Error Logging Facility

Technical User's Guide

March, 1985
BFISD8279P

BFISD 8279

# TABLE OF CONTENTS

Paragraph                                                           Page

PREFACE.............................................  vii

1.0           INTRODUCTION .......................................  1

2.0           FUNCTIONAL DESCRIPTION .............................  1

3.0           ERROR LOG DISPLAY/PRINTOUT FORMATS .................  2

3.1             BY TYPE Log Format................................  2

3.2             CHRONOLOGICAL LOG Format .........................  2

4.0           OPERATION...........................................  4

4.1             Displaying/Printing the Error Logs ...............  4

4.2             Initializing the CHRONOLOGICAL LOG ...............  6

5.0           FLOPPY DISK.........................................  7

5.1             Minor Field.......................................  7

5.2             Type Field......................................... 7

5.3             Lettered Fields...................................  7

6.0           WINCHESTER DISK.....................................  13

6.1             Minor Field.......................................  13

6.2             Type Field........................................  13

6.3             Lettered Fields...................................  13

7.0           PARITY ERROR........................................  21

7.1             Minor Field.......................................  21

7.2             Type Field........................................  21

7.3             Lettered Fields...................................  21

8.0           TRAP................................................  23

8.1             Name Field........................................  23

8.2             Type Field........................................  26

8.3             Lettered Fields...................................  26

9.0           PRINTER FILTER (PRINTER)............................  29

9.1             Minor Field.......................................  29

9.2             Type Field........................................  29

9.3             Lettered Fields...................................  30

10.0          FOUR WAY............................................  33

10.1            Minor Field.......................................  33

10.2            Type Field........................................  33

10.3            Lettered Fields...................................  34

11.0          CMB (SCC CHIP)......................................  35

11.1            Minor Field.......................................  35

11.2            Type Field........................................  35

11.3            Lettered Fields...................................  35

iii

BFISD 8279

TABLE OF CONTENTS (continued)

Paragraph                                                           Page

12.0          DEV 11,0 (LAN)......................................  37

12.1            Type Field........................................  37

12.2            Lettered Fields...................................  37

13.0          CARTRIDGE TAPE......................................  39

13.1            Type Field........................................  39

13.2            Class Field.......................................  39

13.3            Lettered Fields...................................  40

iv

BFISD 8279

LIST OF ILLUSTRATIONS

Figure                                                               Page

3-1           Sample, BY TYPE Log Format..........................   3

3-2           Sample, CHRONOLOGICAL LOG Format....................   5

9-1           Rear View of Base Unit, Showing Location

of All Ports......................................   30

v/vi

THIS PAGE LEFT

BLANK INTENTIONALLY

BFISD 8279

PREFACE

This manual describes the use of the (software) error logging facility in the

MAI 2000 series Desktop Computer System.  The information in this manual is in-

tended for for use by customer engineers (C.E.'s) in the field.

The major topics covered in this manual are:

PARAGRAPH 1.0           INTRODUCTION

PARAGRAPH 2.0           FUNCTIONAL DESCRIPTION

PARAGRAPH 3.0           ERROR LOG DISPLAY/PRINTOUT FORMATS

PARAGRAPH 4.0           OPERATION

PARAGRAPH 5.0           FLOPPY DISK

PARAGRAPH 6.0           WINCHESTER DISK

PARAGRAPH 7.0           PARITY ERROR

PARAGRAPH 8.0           TRAP

PARAGRAPH 9.0           PRINTER FILTER (PRINTER)

PARAGRAPH 10.0          FOUR WAY

PARAGRAPH 11.0          CMB (SCC CHIP)

PARAGRAPH 12.0          DEV 11,0 (LAN)

PARAGRAPH 13.0          CARTRIDGE TAPE

vii/viii

THIS PAGE LEFT

BLANK INTENTIONALLY

BFISD 8279

MAI 2000

ERROR LOGGING FACILITY

# 1.0  INTRODUCTION

Error logging is a standard feature in the MAI 2000 operating system (BOSS/IX).

It enables the C.E. to print a history of hard and soft errors in the MAI 2000

System.  From an analysis of this printout, he may acquire information useful

in troubleshooting the system hardware.

# 2.0  FUNCTIONAL DESCRIPTION

A software routine writes error information to main memory RAM.  Then, to per.

manently retain a record of  the error, a program that has been monitoring the

logging of error information records the RAM contents to disk.

When a loggable error occurs in the MAI 2000 System, the software routine that

monitors the portion of hardware associated with that error makes a system call

to the Error Logger  routine.  This routine is located in main memory RAM (read/

write random access memory) and is always present and prepared for a call.

The Error Logger routine accepts error information from callers and places it

into a dedicated area in main memory RAM.  This area will hold a total of eight

such entries.

Another routine, the Error Log Program, automatically starts when the user ini-

tiates the multiuser mode.  The program periodically transfers error informa-

tion in RAM to a disk log file on a (Winchester) disk.  In addition to updating

that file, it displays/prints the information in the file and the current error

information located  in  RAM.  (The program is located in the user area of main

memory RAM, and hence may be swapped between RAM and disk.)

Two catagories of error information may be recorded in the disk log, in two

distinct sections:

•  By type (BY TYPE LOG)

•  By occurrence (CHRONOLOGICAL LOG)

The top line of the BY TYPE LOG records the kind (or "type") of error logged

(e.g., disk error, memory parity error, etc.) and the total number of occur-

rences of that type  of  error.  The remaining lines record information about the

most recent occurrence.  Barring file destruction, the error count can remain

in the MAI 2000 system permanently.

The By occurrence section is an optional CHRONOLOGICAL LOG, 100 records long by

default.  Error information is recorded in this section when the error occurs

and is a copy of the main memory RAM entry for that error.

1

BFISD 8279

# 3.0  ERROR LOG DISPLAY/PRINTOUT FORMATS

The BY TYPE log is displayed/printed first and is followed by the CHRONOLOGICAL

LOG.  The CHRONOLOGICAL LOG entries are displayed/printed in the order of the

most recent occurrence first, i.e., the most recent occurrence is at the top of

the display/printout.

The display format of the error log is cryptic; the text identifiers displayed/

printed are described in paragraph 3.2.  The datum following a character is

raw, for example:

A: 428       B: -79     C: FFFF8000

Space is allocated for up to five decimal and three hexadecimal entries and one

4-character text field.  Thus a disk driver (operating system routine control-

ling the disk hardware) may wish to report a block number and track number in

decimal fields A and B; but the  tape driver may use  fields A and B in the tape

error log entry to mean a soft error count and timeout error type.  Hence field

interpretation is tied closely to the kind ("type") of device whose error in-

formation is being logged and thus cannot be generalized.  Specific information

is supplied in paragraphs 5.0 through 13.3

## 3.1  BY TYPE Log Format

The BY TYPE log display/printout is formatted as follows:

Device name   Minor:     First: (day, date, time)        Count:

Following this is a copy of the last entry logged in RAM for that device. (Re-

fer to paragraph 3.2 for a description of the format.)  One display/printout

entry is provided for each "type" of  device.  A sample of the BY TYPE log is

shown in figure 3-1.

•  Device name - the common name of the device,  e.g., floppy  disk, 4-way

controller, etc., followed  by the minor number

•  Minor - provides additional information as to location of error

•  First - first time this error occurred (24-hour clock  format)

•  Count - total count of this type since the Error Log Program was restarted

## 3.2  CHRONOLOGICAL LOG Format

The CHRONOLOGICAL LOG display/printout is formatted as follows:

Device name    Minor:    Type:                (day, date, time)

No.:      No. since boot:       Class:        Rev:

A: decimal    B: decimal    C: decimal   D: decimal   E: decimal

T: hex number U: hex number V: hex number     Z:  text

2

Rev :  8

File:  /etc/error, log                                                   BFISD 8279

Created Wed Feb 13 1985 09:50 26 with 100 records

Updated Tue Feb 26 1983 10:45:04 with error number 54

System Serial Number:  2000-90204

Booted OS  Version :  EOS7139C,  BOSS/IX release 7. 1A*34 (Jan 12 1985 05:04)

-----------------------------------------------------------------------------

BY TYPE LOG:

-----------------------------------------------------------------------------

Bus error

Illegal instruction

Bpt/trace

lot

Power fail

System call

Trap instruction

Parity error                First:  Tue Feb 26 1985 10:43:32       Count:  4

Type:  -32767    Tue Feb 26 1985 10:45:04

No :  53)    No  since boot:  20         Class:        Rev. :

A  148          B  0            C:  O            D:  0            E:  0

T: C0000        U.  40          V:  0                 Z:

PIRQ

Floating point

Memory Management

Odd address

Div by 0

Check

Overflow

Privilege

Error halt

Dev 1.0

Dev 2.0

Dev 3.0

Dev 4.0

Parallel printer Minor:   0

Cartridge tape   Minor:   O

Floppy disk      Minor:   0

Printer filter   Minor:   0

CMB              Minor:   0   First:  Mon Feb 25 1985 16:14:43       Count:  16

Type:  1         Mon Feb 25 1985 16:16:28

No. :  50     No  since boot:  17         Class:        Rev :

A  0            B:  0            C:  0            D:  0            E:  0

T: 10           U:  O            V:  0                 Z:

Four way         Minor:   0

Dev 11.0

Dev 12.0

RMI disk         Minor:   0

Winchester disk  Minor:   0

Figure 3-1.   Sample, BY TYPE Log Format

3

BFISD 8279

The fields identified by titles are defined as follows:

•  Device name - the common name of the device, e.g., floppy disk, 4-way

controller, etc., followed by a Minor number when required

•  Type - a decimal number, usually a system error

•  Day Date Time - the time the error occurred, presented in a 24-hour clock

format

•  No. - the nth logged error since the CHRONOLOGICAL LOG was initialized

•  No. since boot - the nth logged error since the system was last booted

•  Class - a character whose meaning is specifically defined by the device/

failure type, e.g., H or S could be used to differentiate between fatal

Qiardware) errors and recoverable (soft) errors

•  Rev - a revision letter used if necessary to identify a change in the

entry meaning from one release level to the next

The fields identified by letters are device/failure specific.  They may mean a

count, an operation code, device status, etc.  To interpret these, refer to

paragraphs 5.0 through 13.3.  A sample of the CHRONOLOGICAL LOG is shown in

figure 3-2.

# 4.0  OPERATION

The following paragraphs discuss the display and printing of the BY TYPE and

CHRONOLOGICAL error logs and the initialization of the CHRONOLOGICAL LOG.

## 4.1  Displaying/Printing the Error Logs

The Error Log Program is automatically initiated when the system multi-user

mode is selected.  To run the program (log errors) in the single-user mode,

type the following:

/sys/errlog /etc/error.log -quiet

To display only the eight memory error log entries, type the following:

/sys/errlog |  p

To print only the esight memory error log entries, type the. following:

/sys/errlog | lpr

To display the disk/memory error logs, type the following:

/sys/errlog /etc/error.log | p

4

BFISD 8279

----------------------------------------------------------------------------

CHRONOLOGICAL LOG

----------------------------------------------------------------------------

Parity error                 Typt: -32767     Tot  Ftb  26 1985 10:45 O4

No :  54    No  since boot: 21         Class         Rev.:

A: 146          B: 0            C: 0             DO            E:  O

T: COOOO        U: 20           V: 0                  Z:

Parity error                 Typt: -32767     Tut  Ftb  26 1965 10:45:04

No. :  53     No. since boot: 20         Class:        Rev. :

A: 146          B: 0            C: 0             D:  O            E:  O

T: COOOO        U: 40           V: 0                  Z:

Parity trror                 Typt: -32767     Tut  Ftb  26 1965 10:43:32

No. :  52     No  since boot: 19         Class:        Rev. :

A: 146          B: 0            C: O             D:  O            E:  0

T: COOOO        U: 0            V: 0                  Z:

Parity trror                 Type: -32767     Tut  Feb  26 1985 10:43:32

No. :  51     No. tince boot: 16         Class:        Rev. :

A: 146          B: 0            C: 0             D:  0            E:  O

T: COOOO        U: 40           V: O                  Z:

CMB              Minor:  0    Type:  1          Mon Feb 25 1985 16:16:28

No. :  50     No. since boot: 17         Class.        Rev. :

A: 0            B: 0            C: 0             D:  0            E:  0

T: 10           U: O            V: 0                  Z:

CMB              Minor:  0    Type:  1          Mon Feb 25 1985 16:15:32

No :  49     No. since boot: 16         Class:        Rev. :

A: 0            B: O            C: 0             D:  0            E:  0

T: 10           U: O            V: O                  Z:

CMB              Minor:  0    Type:  1          Mon Feb 25 1985 16:15:32

No :  48     No  since boot  15         Class:        Rtv. :

A: 0            B. 0            C: O             D:  0            E:  0

T. 10           U: 0            V: 0                  Z:

CMB              Minor:  0    Type:  1          Mon Feb 25 1985 16:15 30

No :  47     No. since boot. 14         Class:        Rtv. :

A: 0            B: 0            C: O             D:  0            E:  0

T: 10           U: 0            V: 0                  Z:

CMB              Minor:  0    Type:  1          Mon Feb 25 1985 16:15:17

No. :  46     No. since boot  13         Class         Rtv. :

A: 0            B  0            C: C             D:  O            E:  0

T: 10           U. 0            V. O                  Z:

CMB              Minor:  0    Type   1          Mon Feb 25 1985 16:15:16

No :  45     No  since boot: 12         Class:        Rev. :

A: 0            B: 0            C: 0             D:  0            E:  0

T: 10           U: 0            V: 0                  Z:

CMB              Minor:  0    Typt:  1          Mon Feb 25 1985 16:15:15

No. :  44     No  since boot: 11         Class:        Rtv. :

A  0            B: 0            C: 0             D:  0            E:  0

T: 10           U: 0            V: 0                  Z:

CMB              Minor   0    Type:  1          Mon Ftb 25 1985 16:15:05

No :  43     No. since boot: 10         Class:        Rtv. :

A: 0            B: 0            C: 0             D:  O            E:  0

T: 10           U. 0            V: 0                  Z:

CMB              Minor:  0    Type:  1          Mon Ftb 25 1985 16:15:05

No. .  42     No. since boot: 9          Class:        Rev :

A  0            B: 0            C: 0             D:  0            E:  0

T: 10           U: 0            V: 0                  Z:

CMB              Minor.  0    Type:  1          Mon Ftb 25 1985 16:15:04

No. :  41     No. sinct boot: 6          Class:        Rtv. :

A: 0            B: 0            C: 0             D: 0           E:  0

T: 10           U: 0            V: 0                  Z:

CMB              Minor:  0    Type:  1          Mon Ftb 25 1985 16:14 44

No :  40     No. sinct boot: 7          Class:        Rev. :

A  0            B: 0            C: O             D:  0            E  0

T: 20           U: 0            V: 0                  Z:

Figure  3-2.          Sample, CHRONOLOGICAL LOG Format

5

BFISD 8279

To print the disk/memory error logs, type the following:

/sys/errlog /etc/error.log  | Ipr

The BY TYPE log is displayed/printed first, followed by the CHRONOLOGICAL LOG.

The CHRONOLOGICAL LOG entries are displayed/printed in the order of the most

recent occurrence first, i.e., the most recent occurrence is at the top of the

display/printout.

## 4.2  Initializing the CHRONOLOGICAL LOG

Normally, the Error Log Program initializes the CHRONOLOGICAL LOG.  The LOG is

initialized with default values of  100, for the number of entries in the LOG,

and 0, for the starting count, so that the LOG will start counting errors from

the number 1.

However, the CHRONOLOGICAL LOG can be reinitialized by the user (C.E.):

a.  To change the size of the file

b.  To change the starting  error count, so that new errors will start from

the new count (useful when a new error log file is desired, and the

current file has entries)

Initializing the CHRONOLOGICAL LOG should only be done in the system single-

user mode.

A newly initialized CHRONOLOGICAL LOG starts counting errors from 1.  When the

number of logged errors reaches the errors value (100 by default), the next en-

try overwrites the first.

To change the file size, type the following:

/sys/errlog /etc/error.log -initial -size=(new number of entries desired)

For example, to change the number of entries desired to 50, type the following:

/sys/errlog /etc/error.log -initial -size=50

To change the starting error count, type the following:

/sys/errlog /etc/error.log -initial -errors=(new errors value)

For example, to change the starting number of the next entry to 15, type the

following:

/sys/errlog /etc/error.log -initial -errors=15

Or, you could do both at the same time, by typing the following:

/sys/errlog /etc/error.log -initial -size=50 -errors=15

6

BFISD 8279

# 5.0  FLOPPY DISK

The purpose of this section is to aid the C.E. in interpreting the fields in

the error log display/printout pertaining to the floppy disk subsystem of the

MAI 2000 Series Desktop Computer System.

Since the Floppy Disk Controller hardware is an integral part of the Central

Microprocessor Board (CMB), in the Model 4108 Base Unit, this section provides

additional hardware-related information not included in other sections (which

pertain to plug-in PCBAs).

## 5.1  Minor Field

If the Minor field displays 0 through 31, the error is associated with drive 1;

if the number is 32 through 63, the error is associated with drive  2.

## 5.2  Type Field

The possible error numbers, shown in the Type field, are -63, -86 and -99.

They are interpreted as follows:

-63 — A write-protected diskette was in the drive prior to the error.

-86 — The error was a seek error or a data transfer error.

-99 — The originally-mounted diskette was not found.

## 5.3  Lettered Fields

The single-letter paragraph  titles below represent the nine error log fields

that are headed by those characters (in this  case, only five fields are used).

The corresponding paragraph interprets that field.

A

Field A contains the logical block (decimal) number of the starting block of

data (in main memory RAM) to be transferred to the floppy disk.

B

Field B contains the number of blocks to be transferred to the floppy disk.

T

Field T indicates the status of the floppy disk driver routine prior to the

error. This field contains the hexadecimal representation of the contents of

a software status register in the routine.

7

BFISD 8279                                                 FLOPPY DISK (cont'd)

The bits in that register are interpreted as follows (bit 0 is the least

significant bit):

0 -- Read.  This was the most recent command to the FDC chip.

1 -- Write.  This was the most recent command to the FDC chip.

2 -- Recalibrate.  This was the most recent command to the FDC chip.

3 -- Seek.  This was the most recent command to the FDC chip.

A -- Not used.  This bit is always 0.

5 -- Not used.  This bit is always 0.

6 -- Error.  This bit indicates that an FDC error occurred; the bit is

always set (1) in the error log display/printout.

7 -- Interrupt.  This bit indicates that an interrupt request (INTRQ)

from the FDC chip occurred prior to the error.

For example, if the number in field T is 42 (hex), the binary equivalent is

0100 0010.  The interpretation is that an error occurred (bit 6=1) during a

Write command (bit 1=1).  Interpretation of fields U and V depends on T.

U

Field U indicates the status of the floppy disk controller (FDC) chip prior

to the error.  The driver routine reads the status register inside the FDC

chip to obtain this information. Field U contains the hexadecimal represen-

tation of the contents of that register.

The bits in the register are interpreted as follows (bit 0 is the least sig-

nificant bit):

0 -- Busy.  This bit is always 0.

1 -- Index Pulse/DRQ.  If the most recent command to the FDC chip prior

to the error was a ("Type I") Seek or Recalibrate (Restore) com-

mand (as indicated by bits 3 or 2 set in field T), then this bit

set indicates that the index mark from the floppy drive had been

detectecl_before the error occurred. This bit is an inverted copy

of the IP input to the FDC chip (pin 35) from the drive.

If the most recent command to the chip prior to the error was one

of the ("Type II/III") Write or Read commands (as indicated by bits

1 or 0 set in field T), then this bit is  a copy of the DRQ (Data

Re Quest) output from the FDC chip (pin 38).

When set, the bit indicates that the data register inside the FDC

chip is full (on a Read operation) or empty (on a Write operation).

The bit is reset (0) otherwise.

8

FLOPPY DISK (cont'd)                                                  BFISD 8279

2 -- Track 00/Lost Data.   If the most recent command to the FDC chip

prior to the error was a ("Type I") Seek or Recalibrate (Restore)

command (as indicated by bits 3 or 2 set in field T), then this bit

set indicates that the read/write head was positioned over Track 00

prior to the error.  The bit is an inverted copy of the TROO input

to the FDC chip (pin 3A) from the drive.

If the most recent coomand to the chip prior to the error was one

of the ("Type II/III") Write or Read commands (as indicated by bits

1 or 0 set in field T), then this bit set indicates that the CPU

did not respond to the DRQ (Data ReQuest) output from the FDC chip

(pin 38) in one byte time.

(When high, DRQ indicates that the data register inside the FDC

chip is full [on a Read operation] or empty [on a Write operation];

DRQ is low otherwise.  The CPU needs this signal to know when to

transfer data to or from the data register inside the FDC chip; and

it must complete the operation within the time it takes a byte to

be shifted into or out of the data shift register inside the FDC

chip.)

3 -- CRC Error.  If the most recent command to the FDC chip prior to the

error was a ("Type I") Seek or Recalibrate (Restore) command (as

indicated by bits 3 or 2 set in field T), then this bit set indi-

cates that a CRC error was encountered in the ID field of the re-

corded format.

If the most recent command to the chip prior to the error was one

of the ("Type II/III") Write or Read commands (as indicated by bits

1 or 0 set in field T), and if bit 4 of field U (this field) also

is set, then this bit set indicates that the CRC error was found in

one or more ID fields of the recorded format.

But if bit 4 is not set, this bit set indicates a data field error.

4  -- Seek Error/Record Not Found.  If the most recent command to the FDC

chip prior to the error was a ("Type I") Seek or Recalibrate (Re-

store) command (as indicated by bits 3 or 2 set in field T), then

this bit set indicates that the desired track was not verified.

(When a Seek, Step or Restore [Recalibrate] command is executed, a

verification operation begins at the end of the settling time after

the read/write head is loaded against the disk.  The track number

from the first encountered ID field is compared against the con-

tents of the track register, inside the FDC chip.  If the track

numbers compare and the ID field CRC is correct, then the verify

operation is complete.)

If the most recent command to the chip prior to the error was one

of the ("Type II/III") Write or Read commands (as indicated by bits

1 or 0 set in field T), then this bit set indicates that a desired

track, sector or side was not found.

9

BFISD 8279                                                  FLOPPY  DISK (cont'd)

5 -- Head Loaded.  This bit can be set only if the most recent command

to the FDC chip prior to the error was a ("Type I")  Seek or Recali-

brate (Restore) command (as indicated by bits 3 or  2 set in field

T).

This bit set indicates that the read/write head was loaded and en-

gaged when the error occurred.

(The bit is a logical AND of the HLD [Head LoaD] and HLT [Head Load

Timing] signals from the FDC chip [pins 28 and 23J.)

6 -- Write Protect.  If the most recent command to the FDC chip prior to

the error was a ("Type I") Seek or Recalibrate (Restore) command

(as indicated by bits 3 or 2 set in field T) or one  of the ("Type

II/III") Write commands (as indicated by bit 1  set  in  field T),

then this bit set indicates that a write-protected  disk was in  the

drive at the time of the error.

7 -- Not Ready.  This bit is set whenever the READY input to the FDC

chip (pin 32) is low.

Example: Assume that the most recent command to the FDC chip was a ("Type II/

III") Write or Read command, as indicated by bits 1 or 0 set in field T.  If

the number in field U is 18 (hex), the binary equivalent is 0001 1000.  The

interpretation is that a CRC error (bit 3=1) was found in one or more ID

fields of the recorded format (bit 4=1).  (Note: if bit 4=0, then the CRC

error is in a data field.)

V

Field V indicates the most recent control command sent to the floppy disk

controller (FDC) prior to the error.  This field contains the hexadecimal

representation of the contents of a software command register in the FDC

driver routine.

The bits in that register are interpreted as follows (bit 0 is the least sig-

nificant bit):

NOTE

The default, or normal, value for Field  V is A

(binary equivalent: 0000 1010).  If any  other

commands are represented in this field,  someone

has entered them for some reason.  To enter the

default commands, refer to the following paragraph.

0 — Not used (bit is always 0)

1 — Retries Enabled

10

FLOPPY DISK (cont'd)                                                  BFISD 8279

2 — Not used (bit is always 0)

3 — Disable Error Messages

4 — Enable Write Verification

5 — Not used (bit is always 0)

6 — Not used (bit is always 0)

7 — Not used (bit is always 0)

For example, if the number in field V is 18 (hex), the binary equivalent is

0001 1000.  The interpretation is that Retries was enabled, and Write Verifi-

cation was disabled.  These are not default commands.  To enter the default

commands, do the following:

NOTE

The following operation can be done only if

the (software) Tool Command Set is installed.

Otherwise,

ADMIN> control -help

Can't execute 'control.'  File does not exist.

will be displayed.

1.  Type 'control -help*.  The usage will appear on the screen as follows:

-c  toggle ECC correction

-e  toggle error messages

-o  toggle seek optimization

-r  toggle retries

-s  set to driver defaults

-v  toggle write verify

2.  Type 'control /dev/(device name) -(letter of desired toggle command).'

Example:

ADMIN> control /dev/wdO -s

Control now set to:

automatic retries

no driver error messages

no write verification

Old value was:

no retries

driver error messages

no write verification

11/12

THIS PAGE LEFT

BLANK INTENTIONALLY

BFISD 8279

# 6.0  WINCHESTER DISK

The purpose of this section is to aid the C.E. in interpreting the fields in

the error log display/printout pertaining to the hard (Winchester) disk subsys-

tem of the MAI 2000 Series Desktop Computer System.

Since the Winchester Disk Controller hardware is on a printed circuit board as-

sembly (PCBA) that plugs into the Central Microprocessor Board (CMB) , in the

Model 4108 Base Unit, this section provides only the necessary information to

isolate a fault to that PCBA or to the Winchester drive.

## 6.1  Minor Field

If the Minor field displays 0 through 31, the error is associated with drive 1;

if the number is 32 through 63, the error is associated with drive 2.

## 6.2  Type Field

The only possible error number, shown in the Type field, is -2.  It is inter-

preted as an I/O (input/output) error.

## 6.3  Lettered Fields

The single-letter paragraph  titles below represent the nine error log fields

that are headed by those characters (in this  case, only six fields  are used).

The corresponding paragraph interprets that field.

A

Field A contains the logical block (decimal) number of the starting block of

data (in main memory RAM) to be transferred to the Winchester disk.

B

Field B contains the number of blocks to be transferred to the Winchester

disk.

C

Field C contains a 1 or a 0.  A 1 in this field means that there were some

problems in the transfer of data to the disk, but, after a few retries, the

transfer was successfully completed.  A 0 in this field means that, after a

specified number of retries,  the data was not successfully transferred.

This error log entry should be preceded by another Winchester disk error log

entry whose C field contains a 1.  If it is not, then the Retry function

probably was turned off by the user (refer to paragraph  V).

13

BFISD 8279                                             WINCHESTER  DISK (cont'd)

T

Field T indicates the most recent status of the Winchester disk driver rout-

tine prior to the error.  The field contains the hexadecimal representation

of the contents of a software status register in the routine.  The bits in

that register represent the most recent command to the controller prior to

the error.  The contents of the register are interpreted as follows (codes 32

through FF are not used):

0 -- Test Unit Ready.  This command tests the status of the drive(s).

If the drive is on line and ready, a "zero status" will be re-

turned in the command completion status  byte.  If the drive is not

ready, the "Check condition" bit will be set in  the command com-

pletion status byte.  A Request Sense command may then be issued

to return the SCSI error code.

1 -- Rezero Unit.  This command sets the selected drive to track zero

and returns the command completion status byte.  If the "Check

condition" status bit is set, a Request Sense command will return

the applicable SCSI error code.

2 -- Not used.

3 -- Request Sense.   This command is valid when the "Check condition"

bit is set in the command completion status byte.  The command

will return the Sense bytes containing the SCSI error codes and,

when applicable, the logical block address of the bad data.

4 -- Format Unit.  Upon receipt of this command, the Formatter causes

the drive(s) to write from index to index all ID and data fields

with a pattern of $6C6C$ and with a block (sector) size of 512

bytes, as specified in an immediately preceding Mode Select com-

mand.  Note that the Format Unit command uses a defect list to

skip bad sectors in a track when formatting the drives.  This list

is compiled by a previous Translate command.

5-7 — Not used.

8 -- Read.   This command transfers a specified number of data blocks

from the drive to the Controller, beginning at a specified logical

block address.  When the Read command is received, the Formatter,

using drive data stored during the power-up/self-test routine,

will verify a valid logical block starting address and seek the

drive to that address.  When the seek is complete, the Formatter

then reads the starting address data into a FIFO buffer, checks

the ECC (Error Correction Code), and begins the DMA data transfer

to the Controller.

9 -- Not used.

14

WINCHESTER DISK (cont'd)                                              BFISD 8279

A -- Write.   This command transfers a specified number of data blocks

from the Controller to the drive, beginning at a specified logical

block address.  When the Write command is received, the Formatter

will verify a valid logical block starting address and seek the

drive to that address.  When the seek is complete, the Formatter

then transfers the starting address data into the FIFO buffer and

writes the buffered data and its associated ECC onto the addressed

logical sector.

B -- Seek.  Upon receipt of this command, the Formatter, using drive

data stored during the power-up/ self-test routine, will verify a

valid logical block address and seek the drive to that address.

The commmand completion status byte is returned immediately after

seek pulses are issued and head motion is started, thereby freeing

the bus for additional commands prior to seek  completion.  Com-

mands received with a seek in progress will cause a command com-

pletion Busy status to be returned.

This permits the Controller to use the SCSI bus for other proces-

sing while the Formatter waits for a SEEK COMPLETE.  When the seek

is complete, the Formatter returns the command completion status

byte.  Verification of  the ID field is not attempted. It should

be noted that the Formatter uses an implied seek on Read and Write

commands, eliminating the need for a separate Seek command with

each operation.

C-E -- Not used.

F -- Translate.  This command performs a logical address to physical

address translation for a given block address (sector) and reports

its physical location.  The report identifies the actual cylinder

in which the sector resides,  the actual head associated with the

cylinder, and the number of bytes from Index at which the sector

is located.

If there is a data error in the ID field of a given sector, a

"Check condition" will be returned in the command completion

status byte.  It is then necessary to translate the sectors before

and after the targeted sector to determine its location (i.e., a

process of elimination).  The Translate command is also used to

report the physical location of a sector flagged as bad.

10-12 -- Not used.

13 -- Write Buffer.  This command permits the drive Controller to write

1 Kbyte of test data into the FIFO buffer, which can then be read

for diagnostic purposes.

14 -- Read Buffer.  This command permits the drive Controller to read 1

Kbyte of test data from the FIFO buffer for diagnostic purposes.

15

BFISD 8279                                             WINCHESTER  DISK (conf.'d)

15 -- Mode Select.  This command provides the Formatter with a list of

parameters that the Format command will use in formatting the

drive.  As the drive is formatted, the parameters sent with the

Mode Select command are stored on track zero.  On subsequent

power-up Resets, the Formatter will read the parameters from the

drive to perform self-tests, and exercise the drive.  The drive

parameters are specified in 14 bytes of two data blocks that fol-

low the Mode Select conmand.

16-19 -- Not used.

1A -- Mode Sense.  This command permits the operating system to read the

drive parameters stored by the Formatter during a power-up Reset

sequence.

1B -- Start/Stop Unit.  This six-byte command is intended for use with

drives that have a designated shipping or landing zone.  A Start

command is indicated when bit 0 of byte 04 is set; otherwise it is

a Stop command.  A Stop command positions the read/write heads

over the designated landing zone.

1C -- Receive Diagnostic.  TBD

1D -- Send Diagnostic.  This command sets the ECC mode to correct read

errors that are correctable and to set the Check Condition bit in

the command completion status byte for read errors that are un-

correctable.

1E-24 -- Not used.

25 -- Read Capacity.

26-27 -- Not used.

28 -- Read.

29 -- Not used.

2A -- Write.

2B -- Write and Verify.

2C-2E -- Not used.

2F -- Verify.

30 -- Not used.

31 -- Search Data Equal.

Example: If the number in field T is 2B (hex), then the interpretation is

that the most recent command to the Winchester Disk Controller was to Write

and Verify.

16

WINCHESTER DISK (cont'd)                                              BFISD 8279

U

Field U indicates the most recent status of the Winchester Disk Controller

(WDC) prior to the error.  The driver routine reads the status register on

the WDC PCBA to obtain this information, and field U contains the hexadecimal

representation of the contents of that register.

Field U is interpreted as follows (codes 26 through FF are not used):

0 -- No Sense.

1 -- No Index Signal.  This code indicates a probable failure in the

drive interface or in the WDC PCBA.

2 -- No Seek Complete.  This code indicates a probable failure in the

drive.

3 -- Write Fault.  This code indicates a probable failure in the drive.

4 -- Drive Not Ready.  This code indicates a probable failure in the

drive or in the drive interface.

5 -- Not used.

6 -- No Track 0.  This code indicates a probable failure in the drive

or in the drive interface.

7-F -- Not used.

10 -- ID CRC Error.  This code indicates a probable failure in the WDC

PCBA.

11 -- Uncorrectable Data Error.  This code indicates a probable media

error.*

12 -- ID Address Mark Not Found.  This code indicates a probable media

error* or WDC PCBA fault.

13 -- Data Address Mark Not Found.  This code indicates a probable media

error* or WDC PCBA fault.

14 -- Record Not Found.  This code indicates a probable media error* or

WDC PCBA fault.

15 -- Seek Error.  This code indicates a probable media error* or drive

fault.

16-17 -- Not used.

*  Before disk reformatting is undertaken,  it is imperative that either the

customer or a Basic Four analyst save as much of the customer files as pos-

sible. (This is not a C.E. responsibility.)

17

BFISD 8279                                             WINCHESTER DISK (cont'd)

18 -- Data Check Without Retry.  This code indicates a probable media

error.*

19 -- ECC Error During Verify.  This code indicates a probable media

error.*

1A -- Interleave Error.  This code indicates a probable system software

error.*

1B -- Not used.

1C -- Blown Format.  This code indicates a probable unformatted disk.*

1D -- Self Test Failed.  This code indicates a probable WDC PCBA fault.

1E -- Defective Track.  This code indicates a probable media error.*

1F -- Not used.

20 -- Invalid Command.  This code indicates a probable system software

error.

21 -- Illegal Block Address.  This code indicates a probable system

software error.

22 -- Not used.

23 -- Volume Overflow.  This code indicates a probable system software

error.

24 -- Bad Argument.  This code indicates a probable system software

error.

25 -- Invalid Logical Unit Number.  This code indicates a probable sys-

tem software error.

* Before disk reformatting is undertaken, it is imperative that either the

customer or a Basic Four analyst save as much of the customer files as pos-

sible.   (This is not a C.E. responsibility.)

Example: If the number in field T is 15 (hex), then the interpretation is

that  a  Seek error has occurred, probably caused by either the Winchester

drive electronics or the disk (media).

V

Field V indicates the most recent control command sent to the Winchester Disk

Controller (WDC) prior to the error.  This field contains the hexadecimal

representation of the contents of a software command register in the WDC

driver routine.  The bits in that register are interpreted as follows (bit 0

is the least significant bit):

18

WINCHESTER DISK (cont'd)                                             BFISD 8279

NOTE

The default, or normal, value for field V is A

(binary equivalent: 0000 1010).  If any other

commands are represented in this field, someone

has entered them for some reason.  To enter the

default commands, refer to the following paragraph.

0 -- ECC and Retries Disabled

1 -- Retries Enabled, Report ECC

2 -- Queue Optimization Disabled

3 -- Disable Error Messages

4 -- Enable Write Verification

5 -- Not used (bit is always 0)

6 -- Not used (bit is always 0)

7 -- Not used (bit is always 0)

For example, if the number in field V is C (hex), the binary equivalent is

0000 1100.  The interpretation is that Queue Optimization and Retries, Report

ECC were disabled.  These are not default commands.  To enter the default

commands, do the following:

NOTE

The following operation can be done only if

the (software) Tool Command Set is installed.

Otherwise,

ADMIN> control -help

Can't execute 'control.' File does not exist.

will be displayed.

1.   Type 'control -help'.  The usage will appear on the screen as follows:

-c  toggle ECC correction

-e  toggle error messages

-o  toggle seek optimization

-r  toggel retries

-s  set to driver defaults

-v  toggle write verify

19

BFISD 8279                                             WINCHESTER DISK (cont'd)

2.   Type 'control /dev/(device name) -(letter of desired toggle command).'

Example:

ADMIN>  control /dev/wdO -s

Control now set to:

ECC

automatic retries

seek optimization

no driver error messages

no write verification

Old value was:

no ECC

no seek optimization

driver error messages

no write verification

20

BFISD 8279

# 7.0  PARITY ERROR

The purpose of this section is to aid the C.E. in interpreting the fields in

the error log display/printout pertaining to the main memory subsystem of the

MAI 2000 Series Desktop Computer System.

Since the Memory Array RAM chips reside on printed circuit board assemblies

(PCBAs) that plug into the Central Microprocessor Board (CMB) , in the Model

4108 Base Unit, this section provides only the necessary information to isolate

a fault to one of the 12 possible Memory Array PCBA RAM banks. (There are two

128K-byte RAM banks per PCBA.)

## 7.1  Minor Field

The Minor field always displays 7.

## 7.2  Type Field

The only possible error number, shown in the Type field, is -32767.  It indi-

cates that non-existent memory was specified for a system call.

## 7.3  Lettered Fields

The single-letter paragraph titles below represent the nine error log fields

that are headed by those characters (in this case, only three fields are used).

The corresponding paragraph interprets that field.

A

Field A contains the process identification number of the process that was

executing when the error occurred.

T

Field T indicates the (hex) address of the 128K-byte bank that failed.  The

addresses of the 12 possible Memory Array PCBA RAM banks are as follows:

BOARD NO.      LOW BANK ADDRESS      HIGH BANK ADDRESS

1             00000-1FFFF            20000-3FFFF

2             40000-5FFFF            60000-7FFFF

3             80000-9FFFF            AOOOO-BFFFF

4             COOOO-DFFFF            EOOOO-10FFFF

5            100000-11FFFF          120000-13FFFF

6            140000-15FFFF          160000-17FFFF

21

BFISD 8279                                                 PARITY ERROR (cont'd)

(When a main memory parity error occurs, CMB hardware detects the error and

latches address bits 17 through 20 in a parity error register [CMB circuit

symbol 3N].  At the same time, the parity error logic on the CMB generates a

level 7 interrupt, and control is transferred to the appropriate interrupt

service routine.  The routine reads the bits in the parity error register and

stores them in a software register.  The contents of this register are dis-

played in field T [hexadecimal representation with four 0's appended] as the

beginning address of the 128K-byte RAM bank {in a Memory Array PCBA] that

produced the parity error.)

U

Field U contains additional parity error information.  This field contains

the hexadecimal representation of the contents of a hardware parity error

register on the CMB.  (This is the same CMB register that contains the ad-

dress information.)

The bits in this register are interpreted as follows (bit 0 is the least

significant bit):

0 -- If this bit is a 1, the parity error occurred on an access by a

peripheral device controller board on the system I/O bus.

If this bit is a 0, the parity error occurred on an access by the

68010 microprocessor, on the CMB.

1 -- Not used.  This bit is always 0.

2 -- Not used.  This bit is always 0.

3 -- Not used.  This bit is always 0.

4 -- Not used.  This bit is always 0.

5 -- If this bit is a 1, the parity error occurred on an access to the

lower (odd order) byte of a word.  Note: This bit is valid only for

the -2 CMB artwork; the bit may be 1 or 0 for the -1 artwork.

6 -- If this bit is a 1, the parity error occurred on an access to the

upper (even order) byte of a word.  Note: This bit is valid only

for the -2 CMB artwork; the bit may be 1 _or 0  for the -1 artwork.

7 -- If this bit is a 1, the 68010 was in the supervisor mode when the

error occurred.

If this bit is a 0, the 68010 was in the user mode when the error

occurred.

For example, if the number in field U is 80 (hex), the binary equivalent is

1000 0000.  The interpretation is that the parity error occurred during a

memory access by the 68010 (bit 0 = 0) in the supervisor mode (bit 7=1).

22

BFISD 8279

# 8.0  TRAP

The purpose of this section is to aid the C.E. in interpreting the fields in

the error log display/printout pertaining to the kinds of errors that result in

traps to the operating system of the MAI 2000 Series Desktop Computer System.

A trap is both an unprogrammed conditional jump, activated by hardware, and a

programmed unconditional jump, using 68010 TRAP instructions.  Initiation of a

trap causes a 68010 exception, and exception processing is performed in much

the same manner as interrupt processing.

## 8.1  Name Field

The possible TRAP error names, along with their interpretation, are as follows:

NAME                                     DESCRIPTION

Bus error            Software or hardware error.  A reference was made to

an address outside the boundaries of physical memory

(installed RAM) address space.

(When an attempt is made to read or write main memory

RAM, a Data  Transfer Acknowledge [PDTACK-] signal must

be returned to the 68010 from the Memory Array PCBA

before a specified timeout;  otherwise, a BERRF- [Bus

ERRor Flag] signal is  returned to the  68010, and  [ex-

ternally generated] exception processing will be

initiated.)

Illegal instruction  User software error.  An attempt was made to execute

an instruction that is not included in the 68010

instruction set.

(If an opcode is fetched whose bit pattern is not that

of one of the defined  instructions, then  [internally

generated] exception processing  will be initiated.)

BPT/trace            Not an error.   A system call was made to the debugger

program in the operating system.

Iot                  Not used.

Power fail           Self-explanatory

(When power supply failure is imminent, an INT7-

[INTerrupt level 7, the non-maskable interrupt] is

generated on the Central Microprocessor Board, and

control is transferred to a handler routine.)

System call          User software error.  A user-generated trap instruc-

tion was used to make a call to a routine in the

operating system kernal.

23

BFISD 8279                                                         TRAP (cont'd)

NAME (con't)                             DESCRIPTION (con't)

Trap instruction     User software error.  When a trap instruction is pro-

cessed, program execution commences at another address.

That address is contained in an exception vector, which

is a memory location in low RAM memory.

This vector can be one of 16  trap vectors. Currently,

only  A vectors are being used:

- syscall    -  bpt   -  syscallregs    - nmilv7

Any reference to the remaining 12 vectors will result in

a trap instruction error.

Parity error         Hardware fault.  A separate error log entry should exist

for this error. See "PARITY," paragraph 7.

PIRQ                 Not used.

Floating point       Not used.

Memory management    User software error. One of the following errors has

occurred:

NOTE

Each of the following six violations, when detected,

causes the following three major events to occur, in

the order presented.

• A write cycle to main memory RAM  is prevented.  As

a result, there  is no return of the data transfer

acknowledge [PDTACK-] signal from the Memory Array

board to the  68010.

• This causes a bus error and forces the 68010 to

process an exception; control is transferred to the

appropriate handler routine in system software.

• The handler routine then reads a status register on

the Central Microprocessor Board to determine that

the bus error originated in the Memory Management

Unit.

- a  Write to a read-only segment

- an instruction fetch from a data segment

- an access to a non-existent segment

24

TRAP (cont'd)                                                         BFISD 8279

NAME (con't)                            DESCRIPTION  (con't)

Memory management     - an access to an address beyond the length of a

(cont'd)                segment

- an access to an address beyond the end of a stack

- an access to an address in the warning area of a

stack

Odd address          User software error.  An attempt was made to reference

a word (or a long word) at an odd address.

Divide by 0          User software error.  This occurred during an arith-

metic operation using the DIVS or DIVU instruction.

As the instruction was executing, an attempt was made

to divide a number by zero, an illegal operation.

Check                User software error.  As a consequence of  the use of

the CHK instruction (which allows a program to check

whether the contents of a 68010 data register are less

than zero or equal to some predetermined value), in-

ternally generated exception (trap) processing was

initiated (indicating the register that was checked

failed the CHK test).

Overflow             User software error.  As a consequence of  the use of

the TRAPV instruction (which allows a program to check

whether the 68010 status register overflow flag bit is

set), internally generated exception (trap) processing

was initiated (indicating an arithmetic result greater

in magnitude than that which can be represented in a

68010 data register).

Privilege violation  User software error.  An attempt was made by the user

program to execute one of the following privileged

instructions.

NOTE

The following instructions are reserved for use only

in the 68010 supervisor mode; an attempt to execute

them in the user mode initiates internally generated

exception [trap] processing.

- STOP

- RESET

- RTE

- MOVE to SR

25

BFISD 8279                                                         TRAP (cont'd)

NAME (con't)                             DESCRIPTION (con't)

Privilege violation   - AND (word) Immediate to SR

(cont'd)

- EOR (word) Immediate to SR

- OR (word) Immediate to SR

- MOVE USP

Errorhalt             Fatal system (hardware/software) error; cpu is halted.

All the foregoing errors can occur when the processor is in the user mode,  ex-

cept Errorhalt.

To ensure the protection of users' programs, the system cannot allow a user to

enter the system software partition of main  memory (whether accidentally or

maliciously).  To this end, the 68010 processor defines certain instructions to

be privileged instructions and determines two separate modes of operation: the

user mode and the supervisor mode.  A bit in the 68010 status register indi-

cates the current processor mode.

The 68010 allows privileged instructions to be executed only in the supervisor

mode.  If an attempt is made to execute a privileged instruction in the user

mode, the 68010 does not execute it but  treats  it as an  illegal instruction and

traps to the operating "system.  The operating system software executes only in

the supervisor mode; a requested address is the actual physical location in RAM

memory, and any address in memory may be referenced.

The user mode, on the other hand, provides the execution environment for the

majority of applications programs. Under the user mode, logical addresses re-

quested by the user are mapped (by memory management unit [MMU] hardware) into

the user's allocated segment of physical (RAM) memory.  Any attempt to access

an address outside this memory segment results in a Bus error, and a Memory

management error is logged.

## 8.2  Type Field

The only possible error number, shown in the Type field, is -32767. It indi-

cates that non-existent memory was specified for a system call.

## 8.3  Lettered Fields

The single-letter paragraph titles below represent the nine error log fields

that are headed by those characters (in  this case, only seven fields are used).

The corresponding paragraph interprets that field.

A

Field A contains the process identification number of the process that was

executing when the error occurred.

26

TRAP (cont'd)                                                        BFISD  8279

B

Field B contains a 1, if the 68010 was in the user mode when the error oc-

curred, or a 0, if the 68010 was in the supervisor mode.

C

Field C contains the reference classification of the 68010 at the time of the

error.  This identifies the kind of bus activity that was being performed at

that time.

(When the 68010 makes a reference, it classifies the kind of reference being

made by putting an encoded reference classification number on three Function

Code output lines.  This allows external translation of addresses, control of

access, and differentiation of special processor states, such as interrupt

acknowledge.)

If the error is a Bus error, an Odd address error, or a Memory management

error, field C will contain the following reference classification numbers

(otherwise, field C will contain 0):

0 -- Not used

1 -- User data

2 -- User program

3 -- Not used

4 -- Not used

5 -- Supervisor data

6 -- Supervisor program

7 -- Interrupt acknowledge

D

If the error is a Bus error, an Odd address error, or a Memory management

error, field D will contain a 1, if the type of access was a Read when the

error occurred, or a 0, if the type of access was a Write.  Otherwise, field

D will contain 0.

T

Field T displays the (hex) address contained in the 68010 program counter

(plus 2 to 10 bytes) at the time of the error.

U

If the error is a Bus error, an Odd address error, or a Memory management

error, field U will contain the first word ("opcode") of a 68010 instruction.

27

BFISD 8279                                                        TRAP (cont'd)

This is the instruction that was in progress when the bus error occurred.

Otherwise, field U will contain 0.

V

If the error is a Bus error, an Odd address error, or a Memory management

error, field V will contain the (hex) address that was accessed at the time

of the error.  Otherwise, field V will contain 0.

28

BFISD 8279

# 9.0  PRINTER FILTER (PRINTER)

The purpose of this section is to aid the C.E. in interpreting the fields in

the error log display/printout pertaining to the status of serial and parallel

printers attached to the MAI 2000 Series Desktop Computer System.

## 9.1  Minor Field

The decimal  numbers that can be displayed in the minor field represent the

various ports at the rear of the Model 4108 Base Unit. (Figure 9-1 shows the

location of  the ports.) The 14 possible numbers, along with the names and

numbers of the ports they represent, are as follows:

NUMBER                          PORT

0          Central Microprocessor Board parallel printer port (#2)

1          Central Microprocessor Board serial port (#0)

2          Central Microprocessor Board serial port (#1)

3          4-Way Controller PCBA number one (port #3)

4          4-Way Controller PCBA number one (port #4)

5          4-Way Controller PCBA number one (port #5)

6          4-Way Controller PCBA number one (port #6)

7          4-Way Controller PCBA number two (port #7)

8          4-Way Controller PCBA number two (port #8)

9          4-Way Controller PCBA number two (port #9)

10          4-Way Controller PCBA number two (port #10)

11          4-Way Controller PCBA number three (port #11)

12          4-Way Controller PCBA number three (port #12)

13          4-Way Controller PCBA number three (port #13)

14          4-Way Controller PCBA number three (port #14)

## 9.2  Type Field

The only possible error numbers, shown in the Type field, are -62 and -108.

The interpretation of -62 is that the requested operation has timed out; the

number -108 indicates a printer I/O transmission error has occurred.

29

BFISD  8279                                               PRINTER FILTER (cont'd)

Figure 9-1.  Rear View of Base Unit, Showing Location of All Ports

## 9.3  Lettered Fields

The single-letter paragraph titles below represent the nine error log fields

that are headed by those characters (in this case, only seven fields are used).

The corresponding paragraph interprets that field.

A

Field A contains the current page number.

B

Field B contains the current line number.

C

Field C contains the current column position (usually the end of the line).

D

Field D contains the Basic Four printer protocol. A 1 in this field indi-

cates that the protocol is on;  a 0 indicates that the protocol is off.

30

PRINTER FILTER  (cont'd)                                             BFISD 8279

E

Field E indicates the make (brand) of attached printer associated with the

error.  This field contains the hexadecimal representation of the contents of

a software register in the "printer filter" routine, which monitors printer

I/O.

The bits in this register are interpreted as follows (bit 0 is the least sig-

nificant bit):

BIT       PRINTER MAKE

0        Printronix

1        Tritel

2        Whisper

3        MVP

4        Diablo

5        Okidata  82a

6        Not used (this bit is always 0)

7        Not used (this bit is always 0)

For example, if the number in field E is 20 (hex), the binary equivalent is

0010 0000.  The interpretation is that the attached printer associated with

the error is an Okidata 82a (bit 5=1).

T

Field T indicates the status of the attached printer associated with the

error, at the time the error occurred. This field contains the hexadecimal

representation of the contents of a software register in the "printer filter"

routine, which monitors printer I/O.

The bits in this register are interpreted as follows (bit 0 is the least

significant bit):

BIT       STATUS

0        Power up

1        Default VFC in effect

2        Not used

3        Offline

31

BFISD 8279                                               PRINTER FILTER (cont'd)

BIT (cont'd)        STATUS  (cont'd)

4            VFC error

5            Paper out

6            Busy

7            Not used (this bit is always 0)

For example, if the number in field T is 20 (hex), the binary equivalent is

0010 0000.  The interpretation is that the attached printer associated with

the error is out of paper (bit 5 * 1).

U

Field U indicates the status of the printer driver routine at the time the

error occurred.  This field contains the hexadecimal representation of the

contents of a software register in that routine.

The bits in this register are interpreted as follows (bit 0 is the least

significant bit):

BIT       STATUS

0        Frame error

1        Parity error

2        Receive overrun  error

3        LRC error

4        Buffer overrun error

5        Not used

6        Acknowledge alternator flag

7        Not used (this bit is always 0)

For example, if the number in field U is 01 (hex), the binary equivalent is

0000 0010.  The interpretation is that the attached printer associated with

the error is out of paper (bit 1 = 1).

32

BFISD 8279

# 10.0  FOUR WAY

The purpose of this section is to aid the C.E. in interpreting the fields in

the error log display/printout pertaining to the 4-Way Controller boards in the

Model 4108 Base Unit (in the MAI 2000 Series Desktop Computer System).

Since all the 4-Way Controller hardware is on a printed circuit board assembly

(PCBA) that plugs into the Central Microprocessor Board (CMB), in the Model

4108 Base Unit, this section provides only the necessary information to isolate

a fault to that PCBA, and to the port associated with the error.

## 10.1  Minor Field

The decimal numbers that can be displayed in the minor field represent the

various ports at the rear of the Model 4108 Base Unit. (Figure 9-1 shows the

location of the ports.)  The 11 possible numbers, along with the PCBA number

and the port number, are as follows:

NUMBER                PCBA NUMBER/PORT #

0          4-Way Controller PCBA number one/port #3

1          4-Way Controller PCBA number one/port #4

2          4-Way Controller PCBA number one/port #5

3          4-Way Controller PCBA number one/port #6

4          4-Way Controller PCBA number two/port #7

5          4-Way Controller PCBA number two/port #8

6          4-Way Controller PCBA number two/port #9

7          4-Way Controller PCBA number two/port 110

8          4-Way Controller PCBA number three/port #11

9          4-Way Controller PCBA number three/port #12

10          4-Way Controller PCBA number three/port #13

11          4-Way Controller PCBA number three/port #14

## 10.2  Type Field

The only possible error numbers, shown in the Type field, are -1 and -2.   The

interpretation of -1 is that an attempt was made to access beyond the end of a

file; the number -2 indicates an I/O error has occurred.

33

BFISD 8279                                                     FOUR WAY (cont'd)

## 10.3  Lettered Fields

The single-letter paragraph titles below represent the nine error log fields

that are headed by those characters (in this case, only two fields are used).

The corresponding paragraph interprets that field.

T

Field T contains the status of the SCC (Serial Communications Controller)

chips on board the 4-Way Controller PCBA associated with the error, at the

time the error occurred.  This field contains the hexadecimal representation

of the contents of the status register inside the SCC chip.  The bits in this

register are interpreted as follows (bit 0 is the least significant bit):

BIT              STATUS

0        Not used

1        Not used

2        Not used

3        Not used

4        Parity error

5        Receive overrun error (indicates that a character was received

but not read from the receiver data register inside the SCC

chip, resulting in the loss of one or more characters)

6        CRC/framing error (a framing error indicates the loss of char-

acter synchronization, faulty transmission,  or a break [all

spaces] condition)

7        End of frame

For example, if the number in field T is 10 (hex), the binary equivalent is

0001 0000.  The interpretation is that the SCC chip associated with the 4-

Way Controller error detected a parity error (bit 4 = 1).

U

Field U contains the status of a hardware register on the 4-Way Controller

PCBA associated with the error.  This field contains the hexadecimal repre-

sentation of the contents of that register.  The contents of the register are

interpreted as follows (only two errors are defined):

HEX              ERROR

81       Illegal command (software error)

83       Bus error (hardware error)

34

CMB (SCC CHIP)                                                        BFISD 8279

# 11.0  CMB (SCC CHIP)

The purpose of this section is to aid the C.E. in interpreting the fields in

the error log display/printout pertaining to the serial I/O subsystem hardware

on the Central Microprocessor Board (CMB), in the Model 4108 Base Unit (in the

MAI 2000 Series Desktop Computer System.

## 11.1  Minor Field

The numbers that can be displayed in the minor field represent the two serial

ports at the rear of the Model 4108 Base Unit. (Figure 9-1 shows the location

of the ports.)  If the Minor field displays 0, the error is associated with

port 0; if the number is 1, the error is associated with port 1.

## 11.2  Type Field

The only possible error number, shown in the Type field, is -1.  The interpre-

tation is that an attempt was made to access beyond the end of a file.

## 11.3  Lettered Fields

Of the nine error log fields that are headed by characters, only field T is

used.  Field T contains the status of the SCC (Serial Communications Control-

ler) chip (on board the CMB) associated with the error,  at the time the error

occurred.  This field contains the hexadecimal representation of the contents

of the status register inside the SCC chip.  The bits in  this register are in-

terpreted as follows (bit 0 is the least significant bit):

BIT            STATUS

0        Not used

1        Not used

2        Not used

3        Not used

4        Parity error

5        Receive overrun error (indicates that a character was received but

not read from the receiver data register inside the SCC chip, re-

sulting in the loss of one or more characters)

6        CRC/framing error (a framing error Indicates the loss of character

synchronization, faulty transmission, or a break [all spaces] con-

dition)

7        End of frame

35

BFISD 8279                                              CMB (SCC CHIP) (cont'd)

For example, if the number in field T is 40 (hex), the binary equivalent is

0100 0000.  The interpretation is that the SCC chip associated with the CMB

serial port error detected a CRC/framing error (bit 6 - 1).

36

BFISD  8279

# 12.0  DEV  11,0 (LAN)

The purpose of this section is to aid the C.E. in interpreting  the fields in

the error log display/printout pertaining to the Local Area Network Controller

(LANC) boards in the Model 4108 Base Unit (in the MAI 2000 Series Desktop Com-

puter System).

Since all the LANC hardware is on a printed circuit board assembly (PCBA) that

plugs into the Central Microprocessor Board (CMB), in the Model 4108 Base Unit,

this section provides only the necessary information to isolate a fault to that

PCBA.

## 12.1  Type Field

The possible error numbers, shown in the Type field, are -665, and -670 through

-676.  They are interpreted as follows:

-665 -- Board disable

-670 -- Hardware-generated error/retry count exhausted

-671 -- Length too long for receiving socket

-672 -- Unintialized socket

-673 -- Invalid transmit control length

-674 -- Invalid socket number

-675 -- RCV socket busy

-676 -- Destination host specification bad

## 12.2  Lettered Fields

The single-letter paragraph titles below represent the nine error log fields

that are headed by those characters (in this case, only two fields are used).

The corresponding paragraph interprets that field.

D

Field D displays the conditions for LANC shutdown.  These conditions are

valid only when error -665 occurs (see "Type Field," above).  The three con-

dition numbers, along with the condition name, are as follows:

1 -- Bus error

11 -- LAN write timeout

21 -- Another LAN node (LANC PCBA) has the same address

37

BFISD 8279                                                       DEV 11,0 (LAN)

E

Field E contains the (hexadecimal) address of the LAN node (station) to which

a LAN transmission failed.  This address is valid only for error numbers -670

through -676 (see "Type Field," above).

38

BFISD 8279

# 13.0  CARTRIDGE TAPE

The purpose of this section is to aid the C.E. in interpreting the fields in

the error log display/printout pertaining to the Magnetic Cartridge (tape)

Streamer (MCS) subsystem of the MAI 2000 Series Desktop Computer System.

Since the MCS Controller hardware is on a printed circuit board assembly (PCBA)

that plugs into the Central Microprocessor Board (CMB), in the Model 4108 Base

Unit, this section provides only the necessary information to isolate a fault

to that PCBA or to MCS itself.

## 13.1  Type Field

The possible error numbers, shown in the Type field, are -2, -62, -63, -89, -97

and -98.  They are interpreted as follows:

- 2 -- An I/O error has occurred

-62 -- The requested operation has timed out

-63 -- The cartridge is write protected

-89 -- The streamer is off-line

-97 -- There is no cartridge in the streamer

-98 -- ERDERCON

## 13.2  Class Field

The Class field contains one of three letters: F, N or S; the interpretation of

these characters is as follows:

F -- Fatal error.  This is an MCS Controller error, a tape read/write abort

error or an MCS timeout error.  Only fatal errors are displayed/printed

as a "By Type" log entry.  All fields in this entry are explained in the

following "Lettered Fields" paragraphs.

N -- Non-fatal error.  This is a "no cartridge" error, a write-protected car-

tridge error or a "drive not ready" error.  (Note: An application such

as "backup/restore" may need to be restarted for  these errors.) All

fields in this entry are explained in the following "Lettered Fields"

paragraphs.

S -- Summary of tape statistics.  This is a summary that is supplied whenever

the cartridge is rewound, and is not an error.  Field A displays the

number of 512-byte blocks processed (not including file marks).  Fields

B, C and D are as explained in the following "Lettered Fields" para-

graphs.  Fields E, T and U are not used.

39

BFISD 8279                                               CARTRIDGE  TAPE  (con't)

## 13.3  Lettered Fields

The single-letter paragraph titles below represent the nine error log fields

that are headed by those characters (in this case, only seven fields are used).

The corresponding paragraph interprets that field.

A

Field A displays the (decimal) block number at the time the error occurred.

B

Field B displays the (decimal) number of files processed since the beginning

of the tape.

C

Field C displays the (decimal) number of Underruns returned by the streamer.

E

Field E contains a number that indicates the most recent operation command

sent to the streamer before the error occurred.  The numbers and their inter-

pretation are as follows:

0 -- MCS controller initialize

1 -- Space forware tape

2 -- Rewind

3 -- Retension

4 -- Erase

5 -- Read streamer (drive) status

16 -- Raw read

17 -- Raw write

18 -- Append

T

Field T contains a number that indicates the streamer status at the time of

the error.  The numbers and their interpretation are as follows:

1 -- Power on/reset

2 -- End of recorded tape

40

CARTRIDGE TAPE (con't)                                               BFISD 8279

4 -- Bus parity error

8 -- Beginning of tape

10 -- Marginal block detected

40 -- Illegal command

80 -- A bit is set (1) in the byte that this hexadecimal number represents

100 -- File mark read

200 -- Bad block was not located

400 -- Unrecoverable data error

800 -- End of tape

1000 -- Write protected

2000 -- Catastrophic error

4000 -- Cartridge is not in place

8000 -- A bit is set (1) in the most significant byte of the two bytes that

this hexadecimal number represents (8000)

U

Field U contains a number that indicates the MCS Controller status at the

time of the error.  The numbers and their interpretation are as follows:

1 — Error, streamer (drive) status required

2 — Catastrophic error

4 — Power fail/reset

8 — Wrote to end of tape

10 — Unerased in area of append

20 — Aborted

40 — Write protected

80 — File mark detected

100 — No cartridge in streamer (drive)

200 — Bad data transfer

41

BFISD 8279                                               CARTRIDGE TAPE (con't)

400 — Read to end of tape

800 — Filler sent

1000 — Chain processing terminated

2000 — IOPB parameter terminated

4000 — End of data

8000 — Operation successful

42